There is no significant difference to the output of OpenSES 4.2 to SES 4.1 for input files listed in Table 1. The summary and data are available in “Inferno Check 2021-02-24.xlsx”.

Table 1: Input Files for Comparison

| # | Input File SES 4.1 | Renamed for OpenSES | Description | Post-Processing |
| --- | --- | --- | --- | --- |
| 1 | inferno.inp | inferno4p2.inp | Sample emergency file provided with SES 4.1.* | inferno4p2_to_inferno.xlsx |
| 2 | normal.inp | normal4p2.inp | Sample normal file provided with SES 4.1.* | normal4p2_to_normal.xslx |
| *Note: Abbreviated prints (Form 12) are eliminated to post-process all potential outputs. |  |  |  |  |

# Process

Input files are simulated in their respective solvers, SES41.exe and OpenSES.exe. The individual results are post-processed with Next-Vis. Next-Vis extracts all the simulation output for second-by-second and summaries into a database. The program calculates the difference and percent error of all results from OpenSES to the SES 41.

The formulas for calculating the difference and percent error for every data point is below:

Equation 1: Difference=OpenSES Output-(SES41 Output)

Equation 2: Percent Error=Absolute Value of [DifferenceSES 41 Output]

# Results

The maximum percent error for all values is reviewed. Values exceeding 0.5% are examined in more detail.

## Inferno

The maximum percent errors are below 0.4%, see Table 2. The variation shown is insignificant.

Table 2: Maximum Percent Error for Inferno

| Sheet | Column | Max | Description | Unit |
| --- | --- | --- | --- | --- |
| SSA | Airflow | 0.000437% | Section Air Flow Rate | kCFM |
| 0 | AirVel | 0% | Bulk Air Velocity | FPM |
| SST | AirTemp | 0.058824% | Air Dry-Bulb Temperature | °F |
| 0 | Humidity | 0.320513% | 1 - Humidity Ratio | °F |
| 0 | Sensible | 0% | Total Sensible Heat Load | BTU/sec |
| 0 | Latent | 0% | Total Latent Heat Load | BTU/sec |
| TRA | RTE | 0% | Train route | # |
| 0 | TYP | 0% | Train Type | # |
| 0 | Location | 0% | Train Location | ft |
| 0 | Speed | 0% | Train Speed | mph |
| 0 | Accel | 0% | Train Acceleration | mph/sec |
| 0 | Drag_KN | 0% | Air Drag (KN) | lbs |
| 0 | Drag_COEFF | 0% | Air Drag (COEFF) | # |
| 0 | Tractive | 0% | Tractive Effort | lbs/motor |
| 0 | M_Current | 0% | Motor Current | amps |
| 0 | L_Current | 0% | Line Current | amps |
| 0 | FLY | 0% | Fly Wheel | RPM |
| 0 | M_Eff | 0% | Motor Efficiency | 0 |
| 0 | G_Accel | 0% | Grid Temperature Acceleration | °F |
| 0 | G_Decel | 0% | Grid Temperature Deceleration | °F |
| 0 | H_Gen | 0% | Heat generated | BTU/SEC-FT |
| 0 | H_Reject | 0% | Heat rejected | BTU/SEC-FT |

## Normal

The maximum percent errors is below 0.5% except for four data points, see Table 4. The variation for these four points are in Table 3. These variations are insignificant.

Table 3: Values with Precent Error above 0.5%

| # | Sheet | Column | Description | Unit | Percent Error | Difference | OpenSES | SES41 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SUB | T_Min_T | Time of dry-bulb temperature minimum | Seconds | 0.56% | 1.6 | 287.7 | 286.1 |
| 2 | SUB | W_Max_T | Time of Humidity ratio maximum | Seconds | 0.56% | 1.6 | 289.3 | 287.7 |
| 3 | ECS | TM_L | Latent Heat Load From Trains and Misc | BTU/HR | 100% | -1 | 0 | 1 |
| 4 | ECS | A_L | Latent Heat Load From Airflow | BTU/HR | 1.72% | 8 | 472 | 464 |

Table 4: Maximum Percent Error for Normal

| Sheet | Column | Max | Description | Unit |
| --- | --- | --- | --- | --- |
| SSA | Airflow | 0.012538% | Section Air Flow Rate | kCFM |
| 0 | AirVel | 0.021097% | Bulk Air Velocity | FPM |
| SST | AirTemp | 0.060901% | Air Dry-Bulb Temperature | °F |
| 0 | Humidity | 0.471698% | 1 - Humidity Ratio | °F |
| 0 | Sensible | 0% | Total Sensible Heat Load | BTU/sec |
| 0 | Latent | 0.071839% | Total Latent Heat Load | BTU/sec |
| TRA | RTE | 0% | Train route | # |
| 0 | TYP | 0% | Train Type | # |
| 0 | Location | 0% | Train Location | ft |
| 0 | Speed | 0% | Train Speed | mph |
| 0 | Accel | 0% | Train Acceleration | mph/sec |
| 0 | Drag_KN | 0% | Air Drag (KN) | lbs |
| 0 | Drag_COEFF | 0% | Air Drag (COEFF) | # |
| 0 | Tractive | 0% | Tractive Effort | lbs/motor |
| 0 | M_Current | 0% | Motor Current | amps |
| 0 | L_Current | 0% | Line Current | amps |
| 0 | FLY | 0% | Fly Wheel | RPM |
| 0 | M_Eff | 0% | Motor Efficiency | 0 |
| 0 | G_Accel | 0% | Grid Temperature Acceleration | °F |
| 0 | G_Decel | 0% | Grid Temperature Deceleration | °F |
| 0 | H_Gen | 0% | Heat generated | BTU/SEC-FT |
| 0 | H_Reject | 0.063492% | Heat rejected | BTU/SEC-FT |
| SEG | A_Max | 0.001% | Airflow rate maximum | kCFM |
| 0 | A_Max_T | 0% | Time of airflow rate maximum | seconds |
| 0 | A_Min | 0.002% | Airflow rate minimum | kCFM |
| 0 | A_Min_T | 0% | Time of airflow rate minimum | seconds |
| 0 | A_P | 0.001% | Airflow rate average value - positive | kCFM |
| 0 | A_N | 0.001% | Airflow rate average value - negative | kCFM |
| 0 | V_Max | 0% | Air velocity maximum | FPM |
| 0 | V_Max_T | 0% | Time of air velocity maximum | seconds |
| 0 | V_Min | 0% | Air velocity minimum | FPM |
| 0 | V_Min_T | 0% | Time of air velocity minimum | seconds |
| 0 | V_P | 0% | Velocity average value - positive | FPM |
| 0 | V_N | 0% | Velocity average value - negative | FPM |
| 0 | A_P_P | 0% | Percent Of Time Airflow Is Positive | % |
| 0 | A_P_N | 0% | Percent Of Time Airflow Is Negative | % |
| 0 | V_E | 0% | Outflow velocity allowable before exceedance | FPM |
| 0 | V_EP | 0% | Percentage Of Time Outflow Velocity Exceeds Design Max | % |
| SUB | T_Max | 0% | Dry-bulb temperature maximum | °F |
| 0 | T_Max_T | 0.061795% | Time of dry-bulb temperature maximum | seconds |
| 0 | T_Min | 0% | dry-bulb temperature minimum | °F |
| 0 | T_Min_T | 0.559245% | Time of dry-bulb temperature minimum | seconds |
| 0 | T_P | 0% | Dry-bulb temperature Positive | °F |
| 0 | T_N | 0% | Dry-bulb temperature Negative | °F |
| 0 | W_Max | 0% | 1- Humidity ratio maximum (kg/kg) | lb/lb |
| 0 | W_Max_T | 0.556135% | Time of Humidity ration maximum | seconds |
| 0 | W_Min | 0% | Humidity ratio minimum | lb/lb |
| 0 | W_Min_T | 0.237741% | Time of Humidity ratio minimum | seconds |
| 0 | W_A | 0% | Average Humidity ratio | lb/lb |
| 0 | TAP_1 | 0% | Percentage of Time above temperature - 1 | % |
| 0 | TAP_2 | 0% | Percentage of Time above temperature - 2 | % |
| 0 | TAP_3 | 0% | Percentage of Time above temperature - 3 | % |
| 0 | TAP_4 | 0% | Percentage of Time above temperature - 4 | % |
| 0 | TAP_5 | 0% | Percentage of Time above temperature - 5 | % |
| 0 | TAP_6 | 0% | Percentage of Time above temperature - 6 | % |
| 0 | TA_1 | 0% | Above temperature - 1 | °F |
| 0 | TA_2 | 0% | Above temperature - 2 | °F |
| 0 | TA_3 | 0% | Above temperature - 3 | °F |
| 0 | TA_4 | 0% | Above temperature - 4 | °F |
| 0 | TA_5 | 0% | Above temperature - 5 | °F |
| 0 | TA_6 | 0% | Above temperature - 6 | °F |
| TES | ET | 0% | Propulsion Energy From Third Rail | KWh |
| 0 | EF | 0% | Propulsion Energy From Flywheel | KWh |
| 0 | EA | 0% | Auxiliary Energy Consumed | KWh |
| 0 | ER | 0% | Energy Regenerated | KWh |
| HAS | MWT | 0% | Morning Wall Temperature | °F |
| 0 | EWT | 0% | Evening Wall Temperature | °F |
| 0 | MAT | 0% | Morning Air Temperature | °F |
| 0 | EAT | 0% | Evening Air Temperature | °F |
| 0 | MH | 0% | Morning Humidity Ratio | lb/lb |
| 0 | EH | 0% | Evening Humidity Ratio | lb/lb |
| ECS | TM_S | 0.000292% | Sensible Heat Load From Trains and Misc. | BTU/HR |
| 0 | TM_L | 100% | Latent Heat Load From Trains and Misc. | BTU/HR |
| 0 | SS_S | 0.001653% | Sensible Heat Load From Steady State Sources | BTU/HR |
| 0 | SS_L | 0% | Latent Heat Load From Steady State Sources | BTU/HR |
| 0 | HS_S | 0% | Sensible Heat Load From Heat Sink | BTU/HR |
| 0 | A_S | 0.045489% | Sensible Heat Load From Airflow | BTU/HR |
| 0 | A_L | 1.724138% | Latent Heat Load From Airflow | BTU/HR |
| 0 | EC_S | 0.015921% | Sensible Heat Load From ECS | BTU/HR |
| 0 | EC_L | 0.013596% | Latent Heat Load From ECS | BTU/HR |
| 0 | TR_S | 0.020346% | Estimated Sensible Heat Load From ECS | BTU/HR |
| 0 | TR_L | 0.013596% | Estimated Latent Heat Load From ECS | BTU/HR |
| 0 | TR_T | 0.05743% | Estimated Total Heat Load From ECS | BTU/HR |